# 咖啡打印机故障:

The malfunction of the coffee printer

1 开机闪黄灯

The yellow light is shining when you turn on the printer.

原因1

The first reason :

在打印机开机自检结束前请不要把杯子升起。

Don’t lift the cup before the end of self-inspection

这时杯子下不来，卡住了。

The cup can’t go down , it got stuck.

解决办法：关掉开关3，拨到“0”，再打开开关3拨到“1”，升降部分的电路重启了。按黄色按钮，将杯子降到底部。

然后按圆形黑色按钮1，关掉打印机，黄色灯灭后，再按按钮1，打印机开机，（打印机部分重启了）。打印机开机正常后，黄色灯长亮后，再升起杯子，点打印。

Solution: turn off the switch 3 , then turn on the switch 3,the circuit of the lifting parts restarts. Press the yellow button , the cup goes down to the bottom . then press the round black button 1, turn off the printer, after the yellow light goes off, press the button 1, turn on the printer, (the printer restarts. ) after the yellow light is on always , lift the cup ,  click the “print”.

原因2：墨盒的触点处有墨水，或者没有安装墨盒。

The second reason: there is some ink on the contactor of the ink box, or you don’t install the ink box yet.
解决办法：取下墨盒，清洁墨盒触点，并且清洁打印机墨车触点后，安装墨盒，重新按按钮1关--开打印机，直到黄色灯长亮。

Solution: uninstall the ink box ,clean the contactor of the ink box, and clean the contactor of the head , install the ink box. Then press the round black button to turn off , and then turn on the printer, the yellow light is on all the time.

原因3：打印前杯子没有升起，就直接点打印了，电脑提示缺纸。

The third reason: you begin to print before the cup goes up, computer shows the printer is out of paper.

解决办法：升起杯子后，短按一次1，打印机会自动打印。

Solution: after lifting the cup , short press 1, the printer will print automaticlly.

如何取消电脑中的打印文档

How to cancel the print documents in computer

解决办法：请到“查看现在正在打印什么”中删除打印任务，然后按黑色圆形按钮1关机，然后按按钮1开机，直到黄色灯不闪，再升起杯子，点打印。

Solution; delete the druckauftrag in “ what it is printing”, then press the round black button 1 to turn off printer, then press 1 to turn on , until the yellow stops flashing, lift the cup , then print.

--------setting --------device and the printer-----what it is printintg

打印半张图案

If it shows like above , it may is because the bad contact of the usb, plug the usb then unplug the usb, or change the port of usb , or wifi is not good.

解决办法：

Solution:

普通款：用USB线连接的，重新拔插USB, 如果是笔记本电脑,请拔掉所有USB接口. 打印机的USB线不要外接HUB ,如果是台式机, 要直接接机箱后面。

The ordinary printer(with usb): re-plugged the usb line testing, if you use laptop, plug all the ports of usb, don’t connect to hub, if desktop ,connect to host computer.

WIFI款：在浏览器中登录192.168.1.188(此处IP地址为举例，实际IP地址按之前设置为准) WIFI打印模块，进入“网格”———“无线”，查看WIFI信号百分比，信号建议保持在90%以上。如果信号不够，请调整路由器的位置，清除路由器与打印机之间的遮挡物。

Wifi: log into 192.168.1.188 in browser(this ip is just an example, the real ip is what you have set)wifi print module, into the “gridding”----“wireless”, see the persentage of the wifi, it should keep more then the 90 persent. If the wifi is not good, you should move the router, move away some barrier between the router and the printer.

当竖着打一半时：

It printed half the picture lateral:

原因：纸张设置错误

Reason: some mistakes about the paper setting

解决办法：图案如果是左右方向的半张，请设置纸张尺寸为A4幅面

Solution: if only half the picture lateral, set the size of paper A4

提示卡纸:

It shows “ paper jam”

1 关闭3---打开3

- Turn off 3---turn on 3

2 按下打印机前面黄色按键，让杯子降下去

2.press the yellow button in front of the printer, let the cup down.

3 长按1 让打印机关机，取消电脑内的打印文档。再按1 ，打印机开机

- Long press 1 to turn on the printer, cancel the print documents in computer. Then press 1, turn on the printer.

4 黄灯长亮后，升起杯子，再点打印

4.after the yellow light is on always, lift the cup , then print.

造成这个故障的原因是：打印机开机自检通过后，再升起杯子，点打印。

Reason: after the end of the self-inspection , lift the cup , then print.

如果提示缺纸，黄灯闪烁 ：

If it shows printer is out of paper, the yellow light is flashing:

1 把杯子按上去

- Lift the cup

2 短按1 ，开始打印

- Short press 1, begin to print.